USERS MANUAL

Gizmo's Charger v2.1 with

gillMod v2.3 for

Smart Taper PWM Regulation of

3 x generator input

Description:

Charging:

This is a diversion controller for battery charging from up to 3 generators of either, or a mix of, Solar, Wind or Micro Hydro sources. Charging has 3 operational modes, being BOOST, TAPER and FLOAT.

- In BOOST mode, Full current is run through the battery.

- In FLOAT mode, a Battery Float voltage is maintained by diverting the excess power into a DUMP Load. The diversion method used is Pulse Width Mode (PWM).

- Between Boost and Float is TAPER mode. TAPER mode starts, where Boost mode ends as the battery voltage has reached the Boost Voltage Set-Point (user adjustable). TAPER mode ends and Float mode starts when the battery voltage reaches just under the Float Voltage Set-Point (user adjustable).

Above the Boost Voltage Set-Point, currents up to 20 amps (default) (‘NoTaper Threshold’ - user adjustable) are not modified and are applied to the battery the same as for Boost charging, but over the ‘NoTaper Threshold’ a tapering algorithm derived from the voltage and current is used to divert increasingly larger currents at larger voltages from the battery to the Dump load. This tapering off of the high charge potential maintains a lower battery temperature and reduces gassing there-by prolonging battery life.

Two user controlled modes are available for maintenance purposes, being, Forced BOOST, and Forced Dump.

- In Forced Boost, full available current is applied to the battery regardless of voltage levels. This is suitable as an Equalising charge if generating capacity is high.

- In Forced Dump, Full battery power is applied to the DUMP Load. This may be used as a diagnostic tool to check for correct Dump Load function, enabling faulty elements to be swiftly identified for replacement.

Two Alarm features are integrated with the operational modes, being, Low Battery Alarm and Hi Battery Drain (Overload) Alarm.

- In Low Battery Alarm mode, the control processor enters a sleep state to reduce power usage and every minute a single medium alarm tone sounded.

- In Hi Battery Drain (Overload) Alarm mode, repeated long, brief, long alarm tones are sounded. This signal is the Morse code “K” meaning ‘over to you’.  The signal continues until the overload is reduced.

Control:

Control of this charger is carried out by two PICAXE Microprocessors. The primary is a PICAXE 28X1 located in

the Control Head, and the secondary a PICAXE 08M is located on the Switching Metering and Connection Unit called 'Piglet'.  The sole function of the secondary processor is to meter the output amperage of each of the three generators and send that data to 'Mother Pig'(the Control Head).  'Mother Pig' controls all other charger functions.

Logging:

The Charger Data logged by the PC is; Solar Array output Amps, Wind Generator Output Amps, Micro Hydro output Amps, Total Generated Amps, Amps used by Load, Excess Amps Dumped, Amps in/out of Battery, Battery Voltage, and Code for the present mode of charging.  Data is able to be logged by two programmes; Programming Editor's Terminal feature, or the Windmill RS232 Data Logger (Free internet download). Each Logging programme requires or suits a specific data arrangement.

Data for logging can be sent to the PC by two methods, The Programming Editor's Serial Cable for local connection, or by 433MHz Data Radio Link for remote connection.  The Local Serial connection can send either Programmes format, whilst the Remote Radio Link sends the Windmill Programmes Format.  If needed, the Terminal Programme can also log the Windmill's Format even though the viewed data is all cramped together.

Terminal's Serial Sample:

.  S_15. + W_4. + H_0. = T_19.  . V_14.3.  L_8.  mode-B.  D_0.  B_11

.  S_15. + W_4. + H_0. = T_19.  . V_14.3.  L_8.  mode-B.  D_0.  B_11

.  S_15. + W_4. + H_0. = T_19.  . V_14.3.  L_8.  mode-B.  D_0.  B_11

.  S_15. + W_4. + H_0. = T_19.  . V_14.3.  L_8.  mode-B.  D_0.  B_11

Terminal's Radio Sample:

s15w4h0t19v143l8mBd0b11

s15w4h0t19v143l8mBd0b11

s15w4h0t19v143l8mBd0b11

s15w4h0t19v143l8mBd0b11

Windmill's Serial and Radio samples are numerous (including excel). A text file sample is:

Windmill Logger

File opened at 13:41:44 Thu 13 Mar 2008

File closed at 13:47:14 Thu 13 Mar 2008

Time		a_Mode  b_BattV  c_Solar  d_Wind	e_Hydro  f_Total  g_Load  h_Dump  i_BattA

Secs		text	volts	amps	amps	amps	amps	amps	amps	amps

13:41:48	B	14.3	15	4	0	19	8	0	11

13:41:51	B	14.2	15	4	0	19	8	0	11

13:41:56	B	14.3	15	4	0	19	8	0	11

13:42:02	B	14.3	15	4	0	19	8	0	11

Controls:

- The Main switch Knob sets the manner of charging, either normal 'CHARGE' that uses the Boost, Taper & Float modes, or the user specific modes of Forced Boost and Forced Dump.

- Push button switches for 'MENU' selection and 'UP' and 'DOWN’ when in the menu. This Menu is available after the display shows 'OK'.    5 parameters are adjustable through the Menu.

  - Boost Voltage Set-Point,

  - Float Voltage Set-Point,

  - Low Battery Voltage Alarm Set-Point,

  - No Taper Amp Threshold Value,

  - Data Out, sending method/format. (PicAXE 28X1 only)

- On the earlier 28X version, push button switches 'UP' and 'DOWN’ serve to select the method of sending data out to the logging PC, This feature is changeable at turn on only (before 'OK') and is not selectable under the menu.

- A toggle switch turns on the display's background lighting.

Operation:				TBA

Adjustments:

Calibration_			TBA

Volts_		TBA

Amps_	 	TBA

Contrast_ 	TBA

Programming_ 		TBA

Repair:

There is little the non-technical operator can do in the event of unit failure.

The following should be checked.  1 Amp Fuse (located inside Control unit), and all wires are secure in the various connector strips.